# CompTIA A+ Core 1 (220-1201) — Targeted Study Guide (Copy/Paste)

## How to use this

- Read once end-to-end.

- Then drill the “Must-Memorize” blocks.

- Then do 2–3 mixed practice sets per day so you don’t “forget other domains” while leveling hardware.

## 1) Mobile Devices (Domain 1.0) — What matters most

### 1.1 Laptop hardware (common)

- RAM types: SO-DIMM (laptop), DIMM (desktop).

- Storage: 2.5” SATA SSD/HDD, M.2 SATA, M.2 NVMe (PCIe), sometimes mSATA (older).

- Wireless cards: often M.2 (Key E) now; older mini-PCIe.

- Screens: LCD + digitizer (touch). If touch fails but display works → digitizer.

### 1.2 Mobile accessories & ports

- USB-C can carry: USB data, charging (USB-PD), and often DisplayPort Alt Mode (video).

- Lightning is Apple’s connector (older iPhones/iPads).

- NFC: tap-to-pay, pairing.

- Hotspot / tethering: shares cellular internet to other devices via Wi-Fi/USB/Bluetooth.

### 1.3 Mobile networking & troubleshooting

- MDM (Mobile Device Management): enrolls devices, enforces policies, remote wipe, configuration profiles.

- Wi-Fi antenna placement: if Wi-Fi signal is weak after repair, common culprit is antenna lead not seated or pinched.

- Bluetooth: short-range peripherals, pairing; often interference with 2.4 GHz Wi-Fi.

Must-memorize

- Touch issue only → digitizer.

- Weak Wi-Fi after repair → antenna connector/placement.

## 2) Networking (Domain 2.0) — Enough to crush it

### 2.1 Ports/protocols (Core list you actually see)

- DNS 53 (UDP/TCP)

- DHCP 67/68 (server/client)

- HTTP 80

- HTTPS 443

- SSH 22

- RDP 3389

- SMB 445

- SMTP 25 (server-server), 587 (submission), 465 (legacy SMTPS)

- IMAP 143 / 993 (secure)

- POP3 110 / 995 (secure)

- FTP 21 (control) + 20 (data, active mode)

- SFTP 22 (SSH file transfer)

- TFTP 69 (UDP)

Quick anchors

- 3389 = RDP

- 445 = SMB (“4-files-445” works)

- 143 = IMAP (“I love you” mnemonic)

### 2.2 Wi-Fi standards & frequencies (high yield)

- 2.4 GHz: longer range, more interference, fewer non-overlapping channels (1/6/11).

- 5 GHz: faster, less interference, shorter range.

- 6 GHz (Wi-Fi 6E): fastest/cleanest, shortest range; requires 6E gear.

- 802.11n: 2.4/5 GHz

- 802.11ac (Wi-Fi 5): 5 GHz

- 802.11ax (Wi-Fi 6/6E): 2.4/5/6 GHz

### 2.3 SOHO gear & concepts

- Router: routes between networks (LAN ↔ WAN), often does NAT.

- Switch: connects devices on LAN (Layer 2).

- Access point: provides Wi-Fi to wired LAN.

- Firewall: filters traffic.

- Modem/ONT: ISP handoff (cable/DSL/fiber).

- VLAN: logically separates networks on the same switch infrastructure.

- VPN: encrypted tunnel; remote access or site-to-site.

### 2.4 Internet connections (what they like to test)

- Cable: coax, shared bandwidth, good speed.

- DSL: phone line, distance-sensitive.

- Fiber: fastest/lowest latency, uses ONT.

- Satellite: high latency.

- Cellular: variable, good backup.

- WISP: wireless ISP, line-of-sight.

Must-memorize

- 2.4 vs 5 vs 6 GHz tradeoffs.

- Router vs switch vs AP roles.

## 3) Hardware (Domain 3.0) — Your main grind (and the exam’s)

### 3.1 Display types & connectors

Common connectors

- HDMI: common consumer A/V.

- DisplayPort: common PC; supports high refresh.

- USB-C/Thunderbolt: can carry DisplayPort video.

- DVI: older digital.

- VGA: analog legacy.

LCD parts

- Backlight (LED), panel, inverter (older CCFL), digitizer (touch).

### 3.2 Cables, connectors, and “pick the cheapest that meets specs”

Ethernet categories (high yield)

- Cat 5: 100 Mbps (legacy)

- Cat 5e: 1 Gbps (most common) → cheapest for ≥750 Mbps

- Cat 6: 1 Gbps (better crosstalk; can do 10G at short distances)

- Cat 6a: 10 Gbps (better)

- Cat 7/8: specialized/overkill for A+

Fiber connector types (know the shapes/use)

- ST: older, twist-lock, separate Tx/Rx connectors (common in questions)

- SC: square push-pull, common

- LC: small form-factor, common in modern gear

- MTRJ: looks like RJ-style fiber; Tx/Rx in one connector

### 3.3 RAM

- DDR generations: DDR3/DDR4/DDR5 are not interchangeable.

- ECC vs non-ECC: ECC for servers/workstations; detects/corrects errors.

- Dual-channel: install matching sticks for performance.

### 3.4 Storage (they love these “pick the right tech” questions)

HDD

- Cheap, large capacity, slower, sensitive to shock.

SSD

- Fast, durable, more expensive per GB.

- SATA SSD: capped by SATA speed.

- NVMe SSD: uses PCIe; much faster; typically M.2.

RAID basics

- RAID 0: speed, no redundancy.

- RAID 1: mirroring, redundancy.

- RAID 5: parity, needs 3+ drives, redundancy.

- RAID 10: mirrored stripes; speed + redundancy; needs 4+ drives.

### 3.5 Motherboards, CPUs, expansion

Form factors

- ATX: standard.

- MicroATX: smaller, fewer slots.

- Mini-ITX: very small.

Expansion slots

- PCIe x16: GPU

- PCIe x1/x4: NIC, sound, capture, etc.

- PCI (legacy) and AGP (very legacy) — know “AGP = old graphics slot”.

Power connectors

- 24-pin (20+4) ATX: motherboard main power.

- 4/8-pin CPU (EPS12V): CPU power near socket.

- SATA power: drives.

- Molex: legacy peripherals.

- PCIe 6/8-pin / 12VHPWR (modern GPUs): video cards.

### 3.6 BIOS/UEFI & settings (1201 style)

- UEFI is modern replacement for BIOS.

- Settings you actually see:

  - Boot order / boot mode (UEFI vs Legacy)

  - Secure Boot

  - TPM / fTPM

  - Virtualization support (Intel VT-x/AMD-V)

  - Date/time

  - Fan curves / hardware monitoring

- CMOS battery keeps BIOS settings/time when unplugged.

### 3.7 Security hardware (your TPM question)

- TPM (Trusted Platform Module): hardware-based crypto chip, helps with full-disk encryption keys, device identity, secure boot measurements.

- AES is an algorithm, not hardware.

- FDE (full disk encryption) is the feature; TPM is the chip that can support it.

Must-memorize

- “Hardware crypto processor for FDE” → TPM.

### 3.8 Printers (tons of people lose points here)

Laser printer process (memorize order)

- Processing (spool/format)

- Charging (primary charge roller)

- Exposing (laser writes image on drum)

- Developing (toner applied to drum)

- Transferring (transfer belt/roller moves toner to paper)

- Fusing (heat + pressure permanently bonds toner)

- Cleaning (removes residual toner)

Color laser key fact

- Transfer belt is what combines CMYK before applying to paper in one pass.

Common printer parts

- Pickup rollers, separation pads (paper feed)

- Duplexing assembly (double-sided)

Troubleshooting patterns

- Smearing/toner rubs off → fuser issue.

- Repeating marks at regular intervals → drum/roller related.

- Paper jams → rollers, debris, wrong paper, worn pickup.

Must-memorize

- “Combines 4 colors before printing” → transfer belt.

## 4) Virtualization & Cloud (Domain 4.0) — Your current weak spot

### 4.1 Virtualization: what it is + why we use it

- Virtualization lets one physical host run multiple virtual machines (VMs).

- Benefits

  - Consolidation (fewer physical servers)

  - Isolation (VMs separated)

  - Snapshots/rollback

  - Testing labs

  - Legacy app support

Type 1 vs Type 2 hypervisor

- Type 1 (bare metal): runs directly on hardware (ESXi, Hyper-V Server). Used in enterprises.

- Type 2 (hosted): runs on top of an OS (VirtualBox, VMware Workstation). Common for learning.

### 4.2 Virtualization requirements (this is exam bait)

Host needs:

- CPU virtualization support: Intel VT-x / AMD-V enabled in BIOS/UEFI.

- Enough RAM (VMs consume it quickly).

- Enough storage (VM disks can be large).

- Sometimes 2nd-level address translation (SLAT) for certain platforms.

### 4.3 Cloud models & terms (you’re strong here, but memorize the mapping)

- IaaS: you manage OS/apps; provider manages hardware.

- PaaS: you deploy apps; provider manages OS/runtime.

- SaaS: you just use the software.

Deployment

- Public, Private, Hybrid, Community.

Cloud characteristics

- Rapid elasticity, on-demand self-service, broad network access, resource pooling, measured service.

Must-memorize

- Type 1 vs Type 2.

- VT-x/AMD-V must be enabled for VMs.

## 5) Hardware & Network Troubleshooting (Domain 5.0) — The “symptom → cause” game

### 5.1 Motherboard/CPU/RAM/Power symptoms

- PC won’t power on: outlet/PSU switch/cable/PSU dead, motherboard short, front-panel header.

- Powers on then off quickly: overheating, bad PSU, CPU fan not detected, short.

- Random reboots: PSU, overheating, RAM issues.

- No POST / beep codes: RAM not seated, GPU, motherboard.

Thermal paste question (classic)

- Boots, then shuts down after a minute → CPU overheats → forgot thermal paste or heatsink not seated.

### 5.2 Storage/RAID issues

- Clicking HDD → failing drive.

- Slow system with HDD but fine CPU/RAM → disk bottleneck.

- RAID degraded → replace failed drive, rebuild.

### 5.3 Video/display/projector issues (super common)

- No display: wrong input source, loose cable, dead monitor, GPU not seated, incorrect resolution/refresh.

- Artifacts/snow: bad cable, failing GPU, wrong refresh rate.

- Dim display: backlight/inverter (older) or brightness settings.

### 5.4 Mobile device troubleshooting patterns

- Swollen battery: replace battery ASAP (safety).

- Overheating: background apps, charging, case, bad battery.

- No cellular: SIM, APN, carrier lock, antenna.

### 5.6 Printer troubleshooting

- Lines down page: dirty drum/rollers.

- Faded prints: low toner, drum, transfer components.

- Paper feed issues: pickup rollers, separation pads.

Must-memorize

- Thermal shutdown after CPU swap → thermal paste/heatsink.

- Printer smear → fuser.

# High-Yield “Memory Page” (Paste into flashcards)

### Ports

DNS 53

DHCP 67/68

HTTP 80

HTTPS 443

SSH 22

RDP 3389

SMB 445

SMTP 25 / 587

IMAP 143 / 993

POP3 110 / 995

FTP 21 + 20

TFTP 69

### Cables & connectors

Cat5 = 100 Mbps

Cat5e = 1 Gbps

ST = fiber twist-lock, separate Tx/Rx

MTRJ = fiber in one (Tx/Rx together)

### Power

24-pin ATX = motherboard

4/8-pin EPS = CPU

PCIe power = GPU

### Laser printer process

Processing → Charging → Exposing → Developing → Transferring → Fusing → Cleaning

Color combine 4 colors = transfer belt

### Virtualization

Type 1 = bare metal

Type 2 = hosted

Needs VT-x/AMD-V enabled + RAM + storage