# 🧠 A+ Core 1 (220-1201) — Motherboard Study Sheets

## STUDY SHEET 1: POWER (Most common failure point)

### 1. ATX 24-pin (Main Power)

What it is:

Primary power to the motherboard (chipset, slots, ports).

A+ tricks:

- System appears dead → 24-pin not fully seated (they click hard).

- Board powers LEDs but won’t boot → CPU power missing (next item).

### 2. EPS 8-pin CPU Power (4+4)

What it is:

Dedicated 12V power to CPU + VRMs.

A+ traps:

- Confusing EPS 8-pin with PCIe 8-pin GPU (keying differs).

- Plugging only the 24-pin → fans spin, no POST.

- High-end boards may have 8+4 or 8+8 — exam may ask when extra power is needed (overclocking / high-TDP CPUs).

## STUDY SHEET 2: CPU + COOLING

### 3. CPU Socket

What it is:

Physical + electrical interface for the processor.

A+ traps:

- Bent pins (LGA) vs bent CPU pins (PGA).

- CPU installed correctly but unsupported BIOS version → no boot.

- Mismatched socket (AM4 ≠ AM5 ≠ LGA1700).

### 4. CPU_FAN Header vs SYS_FAN

What it is:

Dedicated fan header the BIOS monitors.

A+ traps (VERY common):

- Cooler plugged into SYS_FAN → “CPU Fan Error” at boot.

- AIO pump should go to AIO_PUMP or CPU_OPT, not SYS_FAN.

- Fan spins but BIOS halts boot → wrong header.

## STUDY SHEET 3: MEMORY (They love this)

### 5. RAM / DIMM Slots

What it is:

Temporary, volatile memory.

A+ traps:

- Wrong slot population → single-channel instead of dual-channel.

- Mixed speeds/voltages → instability.

- “No POST” fixed by reseating RAM (click on both ends).

- ECC vs non-ECC confusion (consumer boards usually non-ECC).

Exam wording to watch for:

“System boots with one stick but not two”

→ Wrong slots or mismatched DIMMs.

## STUDY SHEET 4: EXPANSION

### 6. PCIe Slots (Physical vs Electrical)

What it is:

Expansion interface for GPUs, NICs, RAID cards, etc.

A+ traps:

- x16 physical slot wired as x4 electrical.

- GPU must go in top PCIe x16 slot (CPU lanes).

- Card fits physically but runs slower → lane limitation.

### 7. Legacy Slots (AGP / PCI)

What it is:

Older expansion standards.

A+ traps:

- AGP ≠ PCIe (they will try to trick you).

- AGP was graphics only.

- PCI is shared bandwidth; PCIe is point-to-point.

## STUDY SHEET 5: STORAGE (Guaranteed exam content)

### 8. SATA Ports

What it is:

Connects HDDs and SATA SSDs.

A+ traps:

- SATA port disabled when M.2 is populated.

- Wrong SATA mode (IDE / AHCI / RAID) after CMOS reset.

- Data vs power cable confusion (SATA power comes from PSU).

### 9. M.2 (NVMe vs SATA)

What it is:

Direct-mount storage interface.

A+ traps (high yield):

- NVMe ≠ SATA M.2 (same slot, different protocol).

- Installing NVMe disables SATA ports.

- Wrong keying: M-key vs B-key.

## STUDY SHEET 6: CHIPSET & ARCHITECTURE

### 10. Northbridge / Southbridge (Legacy)

What it is:

- Northbridge: CPU ↔ RAM ↔ graphics

- Southbridge: I/O (USB, SATA, audio)

A+ traps:

- Modern boards don’t use these terms → replaced by PCH.

- Exam may show an old board and expect recognition.

### 11. PCH (Modern Boards)

What it is:

Single chip handling all non-CPU I/O.

A+ traps:

- CPU lanes vs chipset lanes distinction.

- Too many devices → bandwidth contention.

## STUDY SHEET 7: HEADERS (PBQ FAVORITES)

### 12. Front-Panel Header (Power/Reset/LEDs)

What it is:

Case buttons and LEDs.

A+ traps:

- System “won’t power on” → power switch on wrong pins.

- LEDs not lighting → polarity reversed (only LEDs care).

### 13. USB Headers (Internal)

What it is:

- USB 2.0: 9-pin

- USB 3.x: 19/20-pin

- USB-C front panel: Type-E

A+ traps:

- Case has USB-C but board doesn’t.

- Plugging USB 3 cable into USB 2 header (won’t fit).

- Bent pins on USB 3 header.

### 14. Audio Header (HD_AUDIO)

What it is:

Front panel headphone/mic.

A+ traps:

- AC’97 vs HD_AUDIO mismatch.

- Front audio dead → header unplugged or wrong standard.

## STUDY SHEET 8: FIRMWARE & TROUBLESHOOTING

### 15. BIOS vs UEFI

What it is:

Firmware interface.

A+ traps:

- Legacy vs UEFI boot mode mismatch.

- Secure Boot blocking OS install.

- BIOS update required for new CPU.

### 16. CMOS Battery & Clear CMOS

What it is:

Maintains firmware settings and clock.

A+ traps:

- Wrong system time → dead battery.

- Clearing CMOS resets SATA mode → OS won’t boot.

- Clearing CMOS fixes POST issues after bad overclock.

## STUDY SHEET 9: PHYSICAL INSTALLATION (PBQ HEAVY)

### 17. Standoffs

What it is:

Prevent shorts between board and case.

A+ traps:

- Extra standoff → board short → no power.

- Missing standoff → board flexing/cracking.

### 18. I/O Shield

What it is:

Grounding + port alignment.

A+ traps:

- Metal tabs inside USB/Ethernet ports.

- Ports blocked or misaligned.

## FINAL EXAM STRATEGY (IMPORTANT)

When you see a motherboard question, ask yourself in this order:

- Is this power, signal, or firmware?

- Which exact connector/header is involved?

- Is this a recognition question or a scenario fix?

- What is the most common failure humans make here?